VISVESVARAYA TECHNOLOGICAL UNIVERSITY

BELAGAVI - 590018

A FINAL PROJECT REPORT ON

“CLOUD-INTEGRATED SELF-HEALING & PREDICTIVE
MAINTENANCE SYSTEM FOR COLD-CHAIN LOGISTICS”

Submitted in partial fulfilment of the requirements for the award of the Degree of

BACHELOR OF ENGINEERING

IN

COMPUTER SCIENCE AND ENGINEERING

Submitted by

SAQIB KHAN (1OX22CS125)

VISHAL G (1OX22CS166)

SYED HANNAN RAFI (1OX22CS150)

SHRAWANTH R (1OX22CS132)

Under the guidance of

Prof. Manjula L.
Assistant Professor, Department of CSE

THE OXFORD COLLEGE OF ENGINEERING

Hosur Road, Bommanahalli, Bengaluru - 560068

2025-2026

CERTIFICATE

Certified that the project work entitled “Cloud-Integrated Self-Healing & Predictive Maintenance System for Cold-Chain Logistics” carried out by Saqib Khan (1OX22CS125), Vishal G (1OX22CS166), Syed Hannan Rafi (1OX22CS150), and Shrawanth R (1OX22CS132), bonafide students of The Oxford College of Engineering, Bengaluru, is submitted in partial fulfilment of the requirements for the award of the Degree of Bachelor of Engineering in Computer Science and Engineering of Visvesvaraya Technological University, Belagavi.

| Project Guide | Head of Department | Principal |
| --- | --- | --- |
| Prof. Manjula L. Assistant Professor, CSE | Dr. R. Kanagavalli Professor & HOD, CSE | Dr. Rakesh S. G. Principal |

DECLARATION

We hereby declare that the project work entitled “Cloud-Integrated Self-Healing & Predictive Maintenance System for Cold-Chain Logistics” is our original project work carried out under the guidance of Prof. Manjula L., Department of Computer Science and Engineering, The Oxford College of Engineering, Bengaluru. The work is submitted in partial fulfilment of the requirements for the award of the Bachelor of Engineering degree and has not been submitted elsewhere for the award of any degree or diploma.

| Name | USN | Signature |
| --- | --- | --- |
| SAQIB KHAN | 1OX22CS125 |  |
| VISHAL G | 1OX22CS166 |  |
| SYED HANNAN RAFI | 1OX22CS150 |  |
| SHRAWANTH R | 1OX22CS132 |  |

Place: Bengaluru

Date: __________________

ACKNOWLEDGEMENT

We express our sincere gratitude to the management and faculty of The Oxford College of Engineering for providing the infrastructure and academic environment required for this work. We thank Dr. Rakesh S. G., Principal, and Dr. R. Kanagavalli, Head of the Department of Computer Science and Engineering, for their support. We are especially grateful to our project guide, Prof. Manjula L., Assistant Professor, Department of CSE, for continuous guidance, technical suggestions, review comments, and encouragement throughout the project. We also thank the teaching and non-teaching staff of the department and everyone who directly or indirectly supported the project.

ABSTRACT

Cold-chain logistics is essential for transporting temperature-sensitive products such as pharmaceuticals, vaccines, dairy products, fresh food, and biological materials. Conventional monitoring systems are predominantly reactive. This project develops a cloud-integrated predictive-maintenance and self-healing prototype that combines multi-sensor IoT monitoring, edge control, ensemble machine learning, redundant cooling, and location-aware rerouting.

The final academic prototype uses an ESP32-S3 as the edge controller. Two DS18B20 sensors measure chamber and Peltier hot-side temperature; an SHT31 measures chamber temperature and humidity; an ACS712 monitors primary cooling current; a reed switch detects door state; and a NEO-6M GPS module provides location when real GPS mode is used. A TEC1-12706 Peltier provides primary cooling and a TEC1-12701/12703 provides backup cooling. Both are controlled through a two-channel logic-level MOSFET stage, with a 12 V, 15 A supply for cooling and a separate regulated 5 V supply for the ESP32-S3.

Sensor telemetry is transmitted over Wi-Fi to a backend where data is stored, transformed into predictive features, and evaluated using XGBoost and Random Forest models. A self-healing state machine distinguishes normal, warning, primary-fault, backup-active, critical-failure, and rerouting states. If primary cooling fails, the system disables it and activates backup cooling. If backup cooling cannot maintain the required thermal condition, the software invokes a rerouting module using GPS or clearly labelled simulated coordinates.

The system is a safe low-voltage academic prototype rather than a pharmaceutical-grade refrigeration unit. It demonstrates sensing, telemetry, predictive analytics, autonomous recovery, event logging, visualization, and escalation.

Keywords: Cold-Chain Logistics, ESP32-S3, IoT, Predictive Maintenance, Self-Healing, XGBoost, Random Forest, Peltier Cooling, GPS Rerouting, Edge-Cloud Architecture.

TABLE OF CONTENTS

| Chapter | Title |
| --- | --- |
| 1 | Introduction |
| 2 | Literature Survey and Research Gap |
| 3 | Requirements and Final Prototype Specification |
| 4 | System Architecture and Design |
| 5 | Hardware Design and Fabrication |
| 6 | Firmware and Edge-Control Implementation |
| 7 | Cloud Backend, Database and Communication |
| 8 | Machine Learning and Predictive Maintenance |
| 9 | Self-Healing and Fault Management |
| 10 | GPS and Intelligent Rerouting |
| 11 | Dashboard and User Interface |
| 12 | Module-Wise Implementation Plan |
| 13 | Testing and Validation Plan |
| 14 | Expected Results and Evaluation |
| 15 | Security, Reliability and Safety |
| 16 | Limitations and Future Scope |
| 17 | Conclusion |
|  | References |
|  | Appendices |

# CHAPTER 1 - INTRODUCTION

## 1.1 Background

Cold-chain logistics maintains products within a controlled temperature range from origin to destination. A thermal excursion can compromise product quality, create financial loss, and in pharmaceutical applications create safety and compliance concerns. Modern transport systems can generate large volumes of sensor and location data, but conventional systems frequently use that data only for threshold alarms and post-failure investigation.

## 1.2 Problem Statement

The project addresses delayed detection of cooling degradation, dependence on manual corrective action, lack of automatic backup cooling, and lack of an integrated mechanism for rerouting temperature-sensitive cargo when local recovery fails.

## 1.3 Objectives

Continuously monitor chamber temperature, humidity, cooling current, hot-side temperature, door state and location.

Use ESP32-S3 for sensor acquisition, local control and status display.

Implement primary and backup Peltier cooling with protected MOSFET switching.

Develop XGBoost and Random Forest predictive-maintenance models.

Implement tiered self-healing and critical rerouting.

Store telemetry, predictions and fault events.

Provide a real-time dashboard with history, alerts and map.

## 1.4 Scope

The implementation is a laboratory-scale, low-voltage academic prototype using thermoelectric cooling rather than a compressor/refrigerant loop. It is not intended to certify pharmaceutical or food cargo.

## 1.5 Final Prototype Revision

The Phase-I conceptual design used a generic refrigeration architecture. The final prototype replaces industrial compressor/refrigerant hardware with a safer Peltier-based cooling assembly. Therefore, refrigerant pressure sensing is deliberately excluded while the predictive-maintenance, self-healing, cloud-monitoring and rerouting objectives are preserved.

# CHAPTER 2 - LITERATURE SURVEY AND RESEARCH GAP

## 2.1 Related Work

| Reference | Relevance |
| --- | --- |
| Ren et al. (2023) | Continuous cold-chain sensing; limited predictive intelligence. |
| Hu et al. (2023) | Digital twin and multi-source data fusion for predictive control. |
| Arifin et al. (2026) | Multimodal IoT sensing in mobile cold-chain logistics. |
| Arvind et al. (2025) | XGBoost and Random Forest in IoT logistics applications. |
| Fahrni et al. (2022) | Cold-chain monitoring and compliance challenges. |
| Bai et al. (2023) | Smart cold-chain sensing and traceability technologies. |
| Pajic et al. (2024) | Temperature monitoring with GPS/logistics integration. |
| Aminzadeh et al. (2025) | Multi-sensor predictive maintenance for industrial compressors. |

## 2.2 Research Gap

| Aspect | Existing Approach | Project Contribution |
| --- | --- | --- |
| Monitoring | Limited or single-sensor monitoring | Multi-sensor environmental and equipment-health monitoring |
| Prediction | Thresholds or isolated ML | XGBoost + Random Forest ensemble |
| Recovery | Alert-only | Automatic backup cooling |
| Escalation | Manual decision | Cold-storage rerouting |
| Architecture | Edge-only/cloud-only | Integrated edge + backend + dashboard |

## 2.3 Justification

The project integrates environmental sensing, equipment-health signals, machine-learning prediction, local recovery, cloud traceability and geospatial escalation in one academic prototype.

# CHAPTER 3 - REQUIREMENTS AND FINAL PROTOTYPE SPECIFICATION

## 3.1 Functional Requirements

Acquire sensors at a configurable interval, nominally five seconds.

Validate readings and explicitly report invalid data.

Control primary and backup cooling independently with an interlock.

Provide OLED, LED and buzzer status.

Transmit telemetry to the backend.

Store telemetry, predictions and state transitions.

Run actual XGBoost and Random Forest model inference.

Support automatic fault logic and controlled fault-injection buttons.

Perform rerouting after critical failure.

## 3.2 Final Hardware

| Component | Qty. | Purpose |
| --- | --- | --- |
| ESP32-S3 Development Board | 1 | Main edge controller |
| DS18B20 waterproof | 2 | Chamber and heatsink temperature |
| SHT31 | 1 | Humidity and secondary temperature |
| ACS712 20A | 1 | Primary Peltier current |
| Reed switch | 1 | Door state |
| NEO-6M GPS | 1 | Location; optional simulation fallback |
| TEC1-12706 | 1 | Primary cooling |
| TEC1-12701/12703 | 1 | Backup cooling |
| Cold plate + large heatsink + 12V fan | 1 set | Thermal assembly |
| 2-channel logic-level MOSFET driver | 1 | Cooling switching |
| 12V 15A SMPS | 1 | Cooling supply |
| 5V 2A regulated USB supply | 1 | ESP32 supply |
| Fuses + holders | 2 | Branch protection |
| 0.96-inch I2C OLED | 1 | Local display |
| Buzzer + LEDs + buttons | 1 set | Alarm and fault demo |
| 30x30x30 cm enclosure + 40-50mm EPS | 1 set | Cold chamber |

## 3.3 Electrical Constraints

Never power a Peltier through ESP32, breadboard or thin jumper wire.

Verify MOSFET compatibility with 3.3V logic and continuous Peltier current.

Condition ACS712 output to remain within ESP32-S3 ADC limits.

Fuse each Peltier branch.

Install heatsink and fan before powering a Peltier.

Enclose mains terminals and keep student work on the low-voltage side.

# CHAPTER 4 - SYSTEM ARCHITECTURE AND DESIGN

## 4.1 Architecture

Sensors -> ESP32-S3 edge controller -> local validation/control -> Wi-Fi telemetry -> backend API -> database -> feature engineering -> XGBoost + Random Forest -> ensemble anomaly score -> self-healing state machine -> dashboard and rerouting.

## 4.2 Telemetry Schema

| Field | Meaning |
| --- | --- |
| device_id | Prototype ID |
| timestamp | Acquisition time |
| chamber_temp_c | Chamber temperature |
| heatsink_temp_c | Hot-side temperature |
| humidity_pct | Relative humidity |
| primary_current_a | Primary current |
| door_open | Door state |
| latitude/longitude | GPS or simulation coordinates |
| primary_cooling | Primary output |
| backup_cooling | Backup output |
| system_state | State-machine state |

## 4.3 State Model

| State | Action |
| --- | --- |
| NORMAL | Primary cooling controlled normally |
| WARNING | Corrective control and closer observation |
| PRIMARY_FAULT | Primary disabled |
| BACKUP_ACTIVE | Backup cooling activated |
| CRITICAL_FAILURE | Critical alarm |
| REROUTING | Select compatible cold-storage facility |

# CHAPTER 5 - HARDWARE DESIGN AND FABRICATION

## 5.1 Cold Chamber

A roughly 30 x 30 x 30 cm fabricated enclosure is insulated using 40-50 mm EPS. A gasketed door, inner liner, foam-safe adhesive, silicone sealant and HVAC foil tape reduce heat leakage.

## 5.2 Primary Cooling

TEC1-12706 couples the chamber cold plate to an external high-capacity heatsink. A 12 V fan removes hot-side heat. Thermal compound is applied thinly and the assembly is clamped evenly.

## 5.3 Backup Cooling

TEC1-12701 is preferred; TEC1-12703 is an alternative. It is independently switched. A shared thermal assembly is cost-effective but must be tested because an inactive Peltier can conduct heat.

## 5.4 Sensor Placement

The chamber temperature sensor is positioned away from direct cold-plate contact. The hot-side probe is fixed to the heatsink region. SHT31 is protected from condensation. The reed switch aligns with the door magnet.

## 5.5 Power Distribution

The 12 V supply feeds separately fused cooling branches through MOSFETs. ESP32-S3 uses a separate regulated 5 V supply. Peltier current uses 16-18 AWG wiring and secure terminals.

# CHAPTER 6 - FIRMWARE AND EDGE-CONTROL IMPLEMENTATION

## 6.1 Firmware Modules

| Module | Responsibility |
| --- | --- |
| config | Pins, thresholds and settings |
| sensors | Sensor/GPS acquisition |
| display | OLED and LEDs |
| cooling | MOSFET control and interlock |
| fault_manager | Rules, injection and state transitions |
| network | Wi-Fi/API and retry |
| main | Non-blocking scheduler |

## 6.2 Sampling and Validation

A non-blocking scheduler allows local safety checks to run more frequently than the nominal five-second telemetry period. The firmware detects disconnected sensors, invalid SHT31 reads, GPS no-fix, button bounce and ACS712 zero-offset errors.

## 6.3 Fault Injection

Three labelled buttons support normal/reset, primary-fault injection and critical/backup-fault injection. They are test controls and do not replace automatic sensor-driven or ML-driven detection.

# CHAPTER 7 - CLOUD BACKEND, DATABASE AND COMMUNICATION

## 7.1 Backend

A Python backend can use FastAPI for development. It separates ingestion, validation, persistence, feature engineering, inference, event handling and dashboard APIs. MQTT can be added when broker-based streaming is required.

## 7.2 API

| Method | Endpoint | Purpose |
| --- | --- | --- |
| POST | /api/v1/telemetry | Receive telemetry |
| GET | /api/v1/status/{device_id} | Current status |
| GET | /api/v1/history/{device_id} | Historical telemetry |
| GET | /api/v1/events/{device_id} | Fault/recovery events |
| GET | /api/v1/predictions/{device_id} | ML history |
| POST | /api/v1/reroute | Rerouting request |

## 7.3 Database

| Table | Contents |
| --- | --- |
| telemetry | Sensor and actuator records |
| predictions | XGB/RF/ensemble outputs |
| fault_events | State transitions and recovery |
| warehouses | Facility coordinates/capabilities |
| rerouting_events | Selected facility and reason |

## 7.4 Network Loss

Local cooling and fault response remain active without cloud connectivity. Communication loss is logged separately and must not be interpreted as refrigeration failure.

# CHAPTER 8 - MACHINE LEARNING AND PREDICTIVE MAINTENANCE

## 8.1 Dataset

Training data combines measured prototype runs with clearly identified synthetic augmentation. Measured scenarios include normal cooling, door-open disturbance, primary cooling degradation, hot-side overheating and current anomalies.

## 8.2 Features

| Feature Group | Examples |
| --- | --- |
| Environmental | chamber temperature, humidity |
| Thermal health | heatsink temperature, temperature rate, deltas |
| Electrical health | primary current, current deviation, rolling current |
| Operational | door state/duration, cooling command |
| Temporal/location | hour, speed/location context when used |

## 8.3 XGBoost and Random Forest

XGBoost models non-linear structured sensor relationships using boosted trees, while Random Forest provides a complementary bagging-based model robust to noisy relationships. Hyperparameters are tuned on validation data.

## 8.4 Ensemble

The final anomaly probability combines the two model outputs, preferably with weights derived from validation F1-score. The decision threshold is selected from measured validation performance.

## 8.5 Evaluation

Confusion matrix

Precision

Recall

F1-score

ROC-AUC where applicable

False-positive/false-negative analysis

Inference latency

## 8.6 Integrity Rule

The dashboard displays probabilities only when trained model artifacts are actually loaded and run with the correct feature schema. Hard-coded percentages are not presented as ML predictions.

# CHAPTER 9 - SELF-HEALING AND FAULT MANAGEMENT

| Tier | Trigger | Action |
| --- | --- | --- |
| Tier 1 | Warning/degradation | Corrective control and stabilization check |
| Tier 2 | Primary fault / failed recovery | Primary OFF, backup ON, alert and log |
| Tier 3 | Backup insufficient / critical | Critical alarm and rerouting |

## 9.1 Fault Evidence

Fault decisions should combine evidence where possible: commanded cooling with abnormal current, rising chamber temperature while the door is closed, excessive hot-side temperature, sustained ensemble anomaly risk, or explicit controlled fault injection.

## 9.2 Recovery Verification

After backup activation, the controller evaluates whether chamber temperature stabilizes or moves toward the configured operating range. Recovery is logged only after the stabilization criterion is satisfied.

## 9.3 Audit Trail

Every transition records timestamp, previous state, new state, trigger source, sensor snapshot, ML risk, action and recovery result.

# CHAPTER 10 - GPS AND INTELLIGENT REROUTING

## 10.1 Modes

Real GPS mode uses NEO-6M. Laboratory simulation mode uses predefined coordinates and must be clearly labelled in the dashboard and report.

## 10.2 Facility Selection

On critical failure, the software filters compatible facilities, evaluates distance or route cost, and selects a suitable nearby cold-storage destination. Advanced scoring can incorporate travel time, thermal deterioration and available capacity.

| Warehouse Field | Description |
| --- | --- |
| warehouse_id | Unique ID |
| name | Facility name |
| latitude/longitude | Location |
| temperature_min/max | Capability |
| available | Availability |
| capacity | Optional capacity |

# CHAPTER 11 - DASHBOARD AND USER INTERFACE

Overview: connectivity, system state, chamber temperature, humidity, hot-side temperature, current, door and cooling status.

Live charts: temperature, humidity, current and hot-side temperature over time.

Predictive maintenance: XGBoost, Random Forest and ensemble outputs with timestamp.

Self-healing timeline: warning, fault, backup, recovery and critical escalation.

Map: vehicle, candidate warehouses and selected rerouting destination.

History: telemetry, predictions, alarms and actions.

# CHAPTER 12 - MODULE-WISE IMPLEMENTATION PLAN

| Module | Work | Exit Criterion |
| --- | --- | --- |
| M01 | ESP32 setup | Upload/serial verified |
| M02 | Temperature/humidity | Stable DS18B20 + SHT31 |
| M03 | Door/OLED/alarm | Local UI verified |
| M04 | ACS712 | Safe calibrated current |
| M05 | GPS | Fix or simulation fallback |
| M06 | Primary cooling | Controlled cooling |
| M07 | Backup cooling | Independent backup |
| M08 | Unified firmware | Single telemetry record |
| M09 | Self-healing | State transitions |
| M10 | Backend API | Telemetry received |
| M11 | Database | Data persisted |
| M12 | Dashboard | Live/history views |
| M13 | Dataset/features | Training-ready data |
| M14 | ML models | Saved/evaluated XGB + RF |
| M15 | Integration | Prediction + control + rerouting |
| M16 | Enclosure | Complete prototype |
| M17 | Verification | Acceptance tests passed |

## 12.1 Module Documentation

Objective

Components/software

Architecture

Exact wiring/pin map

Algorithm/flowchart

Implementation

Test procedure

Expected and actual result

Evidence

Problems/fixes

Final status

# CHAPTER 13 - TESTING AND VALIDATION PLAN

| ID | Test | Acceptance Criterion |
| --- | --- | --- |
| T01 | ESP32 boot | Repeated stable boot |
| T02 | DS18B20 | Two valid readings |
| T03 | SHT31 | Valid temperature/humidity |
| T04 | Door | Correct open/close |
| T05 | ACS712 | Calibrated current response |
| T06 | GPS | Valid fix or explicit no-fix |
| T07 | Local UI | OLED/LED/buzzer correct |
| T08 | Primary cooling | Cooling trend measured |
| T09 | Primary fault | Backup requested |
| T10 | Backup cooling | Independent activation |
| T11 | Critical fault | Rerouting triggered |
| T12 | Wi-Fi loss | Local control continues |
| T13 | Backend/DB | Telemetry persists |
| T14 | ML | Real model inference |
| T15 | Dashboard | Live update/history |
| T16 | Rerouting | Compatible facility selected |
| T17 | Restart | Defined safe recovery |

## 13.1 Thermal Characterization

Record ambient temperature, chamber temperature, hot-side temperature, cooling current and door state through a controlled run. Plot chamber temperature versus time and document cooldown, steady state and door-opening recovery.

## 13.2 ML Validation

Use separate training and validation/test data. Report class distribution, split method, confusion matrix and metrics. Synthetic data must be distinguished from measured prototype data.

# CHAPTER 14 - EXPECTED RESULTS AND EVALUATION

Expected behavior is normal primary cooling, predictive risk monitoring, automatic transition to backup after confirmed primary failure, recovery evaluation, and critical rerouting when local recovery is unsuccessful.

| Metric | Final Result |
| --- | --- |
| Ambient temperature | To be measured |
| Initial chamber temperature | To be measured |
| Minimum chamber temperature | To be measured |
| Cooldown time | To be measured |
| Primary current | To be calibrated |
| Backup recovery trend | To be measured |
| XGBoost F1-score | To be measured |
| Random Forest F1-score | To be measured |
| Ensemble F1-score | To be measured |
| API latency | To be measured |
| Rerouting decision time | To be measured |

## 14.1 Result Integrity

Experimental values are not invented before implementation. Measured values, plots, screenshots and model metrics should replace the placeholders after testing.

# CHAPTER 15 - SECURITY, RELIABILITY AND SAFETY

## 15.1 Security

Credentials should not be hard-coded into public repositories. Production deployment should use encrypted transport, authenticated devices, secure secret storage and role-based access.

## 15.2 Reliability

Local control independent of cloud

Explicit sensor-error states

Watchdog/restart strategy

Cooling interlock

Branch fuses

Event logging

Graceful backend retry

## 15.3 Safety

Keep student wiring on low-voltage side.

Enclose mains terminals.

Never route Peltier current through breadboards/jumpers.

Never power Peltier without heatsink/fan.

Use suitable wire, terminals and fuses.

Protect electronics from condensation.

Verify Peltier polarity and hot-side temperature.

# CHAPTER 16 - LIMITATIONS AND FUTURE SCOPE

## 16.1 Limitations

Peltier cooling does not reproduce all compressor/refrigerant failure modes.

Cooling performance depends strongly on insulation and ambient conditions.

ACS712 requires calibration and safe ADC interfacing.

GPS may not fix indoors.

ML performance depends on measured failure data.

Simulation/synthetic data must be labelled.

Prototype is not a certified cold-chain appliance.

## 16.2 Future Scope

Industrial refrigeration integration

Industrial sensors and redundant probes

4G/5G connectivity

Fleet-scale cloud deployment

LSTM/TCN models

Digital twin

Federated learning

Mobile application

Authenticated OTA updates

Industrial enclosure/condensation management

# CHAPTER 17 - CONCLUSION

The project provides an implementation-ready architecture for a Cloud-Integrated Self-Healing and Predictive Maintenance System for Cold-Chain Logistics. The final prototype converts the Phase-I concept into a practical ESP32-S3 and Peltier-based laboratory system integrating sensing, protected primary and backup cooling, edge control, cloud telemetry, persistent storage, XGBoost and Random Forest predictive analytics, autonomous recovery, dashboard visualization, and GPS/simulation-assisted rerouting.

Its principal contribution is the integration of prediction and action. The system is designed to identify developing problems, verify them using physical evidence, attempt local recovery, activate redundant cooling when necessary, and escalate to logistics rerouting when recovery is unsuccessful. Final performance claims will be entered only after physical testing and ML validation.

# REFERENCES

[1] J. Ren et al., “A Self-Powered Sensor Network Data Acquisition, Modeling, and Analysis Method for Cold Chain Logistics Quality Perception,” IEEE Sensors Journal, 2023.

[2] B. Hu et al., “Construction of Digital Twin System for Cold Chain Logistics Stereo Warehouse,” IEEE Access, 2023.

[3] M. Arifin et al., “Integrated Smart Cold Chain: A Dual-Layer IoT Framework for Driver Fatigue and Perishable Cargo Monitoring,” IEEE Access, 2026.

[4] V. R. Arvind et al., “Intelligent Warehousing: A Machine Learning and IoT Framework for Precision Inventory Optimization,” IEEE Access, 2025.

[5] M. L. Fahrni et al., “Management of COVID-19 vaccines cold chain logistics: a scoping review,” Journal of Pharmaceutical Policy and Practice, 2022.

[6] L. Bai et al., “Overview of Food Preservation and Traceability Technology in the Smart Cold Chain System,” Foods, 2023.

[7] V. Pajic et al., “Enhancing Cold Chain Logistics: A Framework for Advanced Temperature Monitoring in Transportation and Storage,” Mechatronics and Intelligent Transportation Systems, 2024.

[8] A. Aminzadeh et al., “A Machine Learning Implementation to Predictive Maintenance and Monitoring of Industrial Compressors,” Sensors, 2025.

[9] T. Chen and C. Guestrin, “XGBoost: A Scalable Tree Boosting System,” ACM SIGKDD, 2016.

[10] L. Breiman, “Random Forests,” Machine Learning, 2001.

Before university submission, verify every bibliographic detail against the original publication.

# APPENDIX A - FINAL BILL OF MATERIALS

| Component | Quantity |
| --- | --- |
| ESP32-S3 Development Board | 1 |
| DS18B20 waterproof | 2 |
| SHT31 | 1 |
| ACS712 20A | 1 |
| Reed switch + magnet | 1 |
| NEO-6M GPS | 1 |
| TEC1-12706 | 1 |
| TEC1-12701/12703 | 1 |
| Aluminium cold plate | 1 |
| Large heatsink | 1 |
| 12V fan | 1 |
| Thermal paste/pads | 1 set |
| Peltier clamps | 1 set |
| 2-channel logic-level MOSFET driver | 1 |
| 12V 15A SMPS | 1 |
| 5V 2A regulated USB supply | 1 |
| Primary/backup fuses | 2 |
| 0.96-inch I2C OLED | 1 |
| Active buzzer | 1 |
| Green/yellow/red LEDs | 3 |
| Push buttons | 3 |
| Resistors | Assorted |
| Breadboard | 1 |
| Perfboard/PCB | 1-2 |
| 16-18 AWG power wire | As required |
| Signal wire/jumpers | As required |
| Terminal blocks/connectors | 1 set |
| Heat-shrink/tape/cable ties | 1 set |
| 30x30x30 cm enclosure | 1 |
| 40-50 mm EPS | As required |
| Door gasket/hinges/latch | 1 set |
| Sealant/adhesive/HVAC tape | 1 set |
| Inner liner | As required |
| Mounting brackets/standoffs | 1 set |
| USB data cable | 1 |
| Laptop | 1 |

# APPENDIX B - FINAL DEMONSTRATION SEQUENCE

| Step | Demonstration | Expected Observation |
| --- | --- | --- |
| 1 | Power ON | ESP32 boots and initializes |
| 2 | Normal monitoring | Live sensor data on dashboard |
| 3 | Primary cooling | Cooling trend recorded |
| 4 | ML monitoring | XGB/RF/ensemble inference |
| 5 | Primary fault | Failure confirmed |
| 6 | Tier 2 | Primary OFF; backup ON; alert |
| 7 | Recovery check | Stabilization evaluated |
| 8 | Critical fault | Backup failure/insufficient recovery |
| 9 | Tier 3 | Rerouting destination selected |
| 10 | Audit review | Complete history visible |

# APPENDIX C - FINAL IMPLEMENTATION CHECKLIST

Exact ESP32-S3 board/pinout documented

Sensors individually tested

ACS712 ADC interface verified safe

OLED/LED/buzzer tested

Primary and backup thermal assemblies tested

MOSFET current rating verified

Fuses installed

High-current wiring inspected

Enclosure insulated/sealed

Unified telemetry generated

Backend and database working

Dashboard working

Measured dataset collected

XGBoost and Random Forest trained/evaluated

Ensemble implemented

Automatic fault detection implemented

Fault injection and backup recovery tested

GPS/simulation mode labelled

Rerouting tested

Wi-Fi-loss behavior tested

Thermal plots and ML metrics inserted

Screenshots/photos inserted

Source code backed up

Final report updated with measured results

# APPENDIX D - ITEMS TO INSERT AFTER IMPLEMENTATION

Prototype photographs

Final circuit diagram with actual GPIOs

Dashboard screenshots

Self-healing timeline screenshot

Map/rerouting screenshot

Measured cooldown and recovery graphs

Confusion matrices and model metrics

Database sample records

Final end-to-end test result table

This report is formatted as a complete final-project framework. Architecture and implementation sections are ready; measured experimental values, photographs, screenshots and final ML metrics should be inserted only after the system is actually tested.