Yes. Below is the complete implementation report of what we have actually built and verified so far, not the future/claimed final system. I’m separating completed software, tested behavior, important code, procedures/commands, and remaining work so you can use this for your project record, guide review, viva preparation, or later final report.

# Implementation Progress Report

## Cloud-Integrated Self-Healing & Predictive Maintenance System for Cold-Chain Logistics

### 1. Project objective

The final system is intended to continuously monitor cold-chain conditions, detect cooling-system degradation/failure, perform automatic recovery using backup cooling, and escalate unrecoverable critical failures to cold-storage rerouting. The final specification also calls for XGBoost + Random Forest predictive maintenance and a real-time dashboard.

At the present stage, the simulator/backend/fault-management/self-healing/control/rerouting software chain is working.

The ML predictive-maintenance models, final dashboard, ESP32-S3 firmware integration, physical sensors, Peltier hardware and physical validation are not yet complete. We must not represent simulated results as hardware measurements; the project specification explicitly requires hardware-dependent results to remain PENDING_HARDWARE.

# 2. Final system architecture being followed

The final hardware architecture is based on:

ESP32-S3

│

├── DS18B20 #1 → Chamber temperature

├── DS18B20 #2 → Peltier heatsink temperature

├── SHT31      → Humidity / secondary temperature

├── ACS712     → Cooling current

├── Reed switch → Door

├── NEO-6M     → GPS

├── OLED

├── LEDs

└── Buzzer

│

▼

Primary TEC1-12706

│

MOSFET Driver

│

Backup TEC1-12701/12703

The authoritative specification uses an ESP32-S3, two DS18B20 sensors, SHT31, ACS712 20A, reed switch, NEO-6M GPS, OLED, buzzer, status LEDs, fault-injection buttons, primary/backup Peltier modules and MOSFET control.

Because the hardware has not yet been physically integrated, we deliberately built a digital simulator first, using the same conceptual telemetry fields expected from the future ESP32.

# 3. Software architecture completed so far

The implemented chain is:

┌──────────────────────┐

│ TELEMETRY SIMULATOR  │

│      CCU-001         │

└──────────┬───────────┘

│ JSON / HTTP

▼

┌──────────────────────┐

│ FastAPI Telemetry API│

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ SQLite Persistence   │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Fault Detector       │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Fault Lifecycle      │

│ + System State       │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Self-Healing Engine  │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Control Command DB   │

└──────────┬───────────┘

│

▼

┌──────────────────────┐

│ Simulator Polls Cmd  │

└──────────┬───────────┘

│

▼

Primary OFF

Backup ON

│

▼

ACK Command

│

▼

BACKUP_ACTIVE

For unrecoverable conditions:

CRITICAL_FAILURE

│

▼

Valid simulated GPS

│

▼

Warehouse Registry

│

▼

Availability Filter

│

▼

Temperature Filter

│

▼

Capacity Filter

│

▼

Haversine Distance

│

▼

Nearest Compatible

Warehouse

│

▼

Rerouting Event

│

▼

REST API

This matches the intended rerouting procedure: find compatible warehouses, remove unavailable ones, check temperature compatibility, calculate distance, select a destination and record the rerouting event.

# 4. Stage 1 — Project environment

Project directory:

E:\cold_chain_project

Main structure:

cold_chain_project/

│

├── backend/

│   ├── api/

│   ├── database/

│   ├── models/

│   └── services/

│

├── simulator/

│

├── database/

│

├── dashboard/

│

├── firmware/

│   └── esp32_s3/

│

├── ml/

│   ├── data/

│   │   ├── raw/

│   │   ├── processed/

│   │   └── synthetic/

│   └── trained_models/

│

├── tests/

│   ├── backend/

│   ├── hardware/

│   ├── integration/

│   └── ml/

│

└── docs/

This follows the intended separation between firmware, simulator, backend, database, ML, dashboard, tests and documentation.

Python environment:

cd E:\cold_chain_project

.venv\Scripts\Activate.ps1

Working prompt:

(.venv) PS E:\cold_chain_project>

Core backend packages installed include FastAPI, Uvicorn, Pydantic, Requests and dotenv support.

# 5. Stage 2 — Digital telemetry simulator

We built:

simulator/

├── simulator.py

└── scenarios.py

Device:

CCU-001

Each simulator run creates a unique:

session_id

and sequential packets:

sequence = 1, 2, 3...

Example telemetry:

{

"device_id": "CCU-001",

"session_id": "1051e690-610e-48c3-ae54-8e9ef7f36d14",

"sequence": 1,

"timestamp": "2026-09-16T18:50:49.930834+00:00",

"telemetry_source": "SIMULATOR",

"simulation_mode": true,

"chamber_temp_c": 16.43,

"heatsink_temp_c": 58.67,

"humidity_pct": 71.66,

"current_a": 0.13,

"door_open": false,

"primary_cooling": false,

"backup_cooling": false,

"gps": {

"latitude": 12.89996,

"longitude": 77.650046,

"valid": true,

"source": "SIMULATED"

},

"scenario": "critical_fault"

}

We deliberately preserve:

telemetry_source = SIMULATOR

simulation_mode = true

gps.source = SIMULATED

so simulated information cannot accidentally be presented as physical measurements. The specification explicitly requires simulation to be clearly labelled.

# 6. Stage 3 — Simulator scenarios

We implemented scenarios including:

normal

door_open

primary_fault

critical_fault

### Normal

Represents healthy operation.

### Door-open

Produces:

door_open = true

and exercises door fault logic.

### Primary fault

Represents primary cooling commanded ON but with abnormal/low current.

Conceptually:

primary_cooling = true

backup_cooling = false

current_a < threshold

### Critical fault

Represents complete cooling unavailability:

primary_cooling = false

backup_cooling = false

chamber_temp_c >= critical threshold

with simulated valid GPS.

# 7. Stage 4 — FastAPI backend

Backend entry point:

backend/main.py

Server command:

uvicorn backend.main:app --host 127.0.0.1 --port 8000 --reload

Local backend:

http://127.0.0.1:8000

Telemetry endpoint:

POST /api/v1/telemetry

The current telemetry API performs storage, fault processing, self-healing and rerouting. The uploaded implementation confirms that telemetry is validated through TelemetryData, persisted using save_telemetry(), processed through process_fault_lifecycle(), and then passed to the self-healing and rerouting services.

# 8. Stage 5 — Telemetry data model

The telemetry model represents information expected from either:

SIMULATOR

or eventually:

ESP32-S3

Important fields include:

device_id

session_id

sequence

timestamp

telemetry_source

simulation_mode

chamber_temp_c

heatsink_temp_c

humidity_pct

current_a

door_open

primary_cooling

backup_cooling

gps.latitude

gps.longitude

gps.valid

gps.source

scenario

This is important because we don't want separate backend logic for simulated and physical telemetry.

Eventually:

Simulator JSON

↓

same schema

↑

ESP32 JSON

# 9. Stage 6 — SQLite database

Development database:

database/cold_chain.db

We have working persistence for the major completed modules.

Important current tables include:

telemetry

fault_events

system_state

control_commands

rerouting_events

The wider final specification eventually calls for additional prediction, warehouse, system-health and testing data structures as ML/dashboard/final validation are completed.

# 10. Stage 7 — Duplicate telemetry protection

One important feature is:

(device_id, session_id, sequence)

being treated as the packet identity.

The API checks whether telemetry was actually newly inserted.

Conceptually:

inserted = save_telemetry(

data=data,

received_at=received_at

)

Then:

if inserted:

fault_result = process_fault_lifecycle(data)

self_healing_result = evaluate_self_healing(

data=data,

fault_result=fault_result

)

This means HTTP retries don't repeatedly generate fault/control actions. The current source explicitly documents this duplicate-protection intention.

# 11. Stage 8 — Rule-based fault detector

We created:

backend/services/fault_detector.py

Current development thresholds include approximately:

CHAMBER_WARNING_TEMP_C = 8

CHAMBER_CRITICAL_TEMP_C = 12

HEATSINK_WARNING_TEMP_C = 55

HEATSINK_CRITICAL_TEMP_C = 70

PRIMARY_MIN_CURRENT_A = 1.0

HUMIDITY_WARNING_PCT = 90

These are development/prototype thresholds, not certified cold-chain operating limits.

Main states:

NORMAL

WARNING

PRIMARY_FAULT

BACKUP_ACTIVE

CRITICAL_FAILURE

# 12. Fault priority logic

The implemented rule hierarchy is conceptually:

if chamber_temp >= critical:

CRITICAL_FAILURE

elif heatsink_temp >= critical:

CRITICAL_FAILURE

elif primary_cooling and current < minimum:

PRIMARY_FAULT

elif chamber_temp >= warning and both_cooling_off:

WARNING / cooling unavailable

elif backup_cooling and not primary_cooling:

BACKUP_ACTIVE

elif chamber_temp >= warning:

WARNING

elif heatsink_temp >= warning:

WARNING

elif door_open:

WARNING

elif humidity >= warning:

WARNING

else:

NORMAL

A particularly important correction was that:

BACKUP_ACTIVE

is an operational recovery state, not another fault.

# 13. Stage 9 — Fault lifecycle manager

We created:

backend/services/fault_manager.py

Its responsibility is not simply to say:

fault = true

It manages the complete event lifecycle:

new fault

↓

create fault_events row

↓

make it active

↓

update system_state

↓

fault continues

↓

reuse existing event

↓

recovery occurs

↓

resolve fault

This prevents a new database fault row from being created for every telemetry packet.

# 14. fault_events

The fault table records information such as:

id

device

fault type

severity

system state

resolved

resolved timestamp

Verified examples included:

Fault #3 PRIMARY_COOLING_CURRENT_FAILURE

Fault #4 CHAMBER_TEMPERATURE_WARNING

Fault #5 PRIMARY_COOLING_CURRENT_FAILURE

Fault #6 CHAMBER_TEMPERATURE_CRITICAL

Resolved primary faults were correctly marked resolved.

Critical fault #6 remained:

resolved = 0

resolved_at = None

while the critical condition continued.

# 15. system_state

Instead of reconstructing the entire current condition from history every time, we maintain the latest system state.

Example verified state:

('CCU-001',

'CRITICAL_FAILURE',

'CHAMBER_TEMPERATURE_CRITICAL',

0,

0)

Meaning:

Device          CCU-001

State           CRITICAL_FAILURE

Active fault    CHAMBER_TEMPERATURE_CRITICAL

Primary         OFF

Backup          OFF

# 16. Stage 10 — Primary cooling current-failure detection

A key fault test was:

Primary commanded ON

+

Current < 1 A

This produces:

PRIMARY_COOLING_CURRENT_FAILURE

with:

severity = HIGH

system_state = PRIMARY_FAULT

This gives us equipment-health detection instead of relying only on chamber temperature.

# 17. Stage 11 — Self-healing engine

We created:

backend/services/self_healing.py

Its role is:

Fault detected

↓

Decide whether automatic action is needed

↓

Create control command

Important command types:

ACTIVATE_BACKUP_COOLING

STOP_ALL_COOLING

RESTORE_PRIMARY_COOLING

For primary current failure:

PRIMARY_COOLING_CURRENT_FAILURE

↓

ACTIVATE_BACKUP_COOLING

↓

primary = OFF

backup = ON

# 18. Control command persistence

We created:

control_commands

A command records:

command ID

device

command type

target primary state

target backup state

status

triggering fault

timestamps

Typical lifecycle:

PENDING

↓

device retrieves command

↓

device applies command

↓

ACKNOWLEDGED

# 19. Command API

The simulator polls the backend for pending commands through the control API.

Conceptually:

GET pending command

↓

apply target state

↓

ACK command

This makes the system closed-loop.

It isn't simply:

Backend says backup should turn on

Instead:

Backend creates command

↓

Device receives it

↓

Device applies it

↓

Device acknowledges it

↓

Next telemetry confirms physical/logical state

# 20. Stage 12 — Simulator command execution

The simulator was upgraded from a simple telemetry generator into a controllable digital device.

When it receives:

ACTIVATE_BACKUP_COOLING

it changes persistent actuator state:

primary_cooling = False

backup_cooling = True

It then produces telemetry reflecting that state.

We also implemented the safety interlock concept:

PRIMARY + BACKUP

cannot both be ON.

This same rule is required in the future Peltier firmware. The final hardware specification explicitly says primary and backup Peltier modules must never be energized simultaneously.

# 21. Primary → backup end-to-end test

We successfully tested:

Primary fault

↓

backend detects low current

↓

PRIMARY_FAULT

↓

self-healing evaluates fault

↓

ACTIVATE_BACKUP_COOLING

↓

command stored

↓

simulator retrieves command

↓

primary OFF

backup ON

↓

ACK

↓

new telemetry

↓

BACKUP_ACTIVE

A verified test included approximately:

Packet 1:

Primary = ON

Backup = OFF

Current ≈ 0.26 A

Command:

ACTIVATE_BACKUP_COOLING

Packet 2:

Primary = OFF

Backup = ON

Current ≈ 2.24 A

This was one of the major software milestones.

# 22. Correct handling of backup recovery

Initially, it would have been easy to treat backup activation as another fault.

We corrected that.

Now:

Primary failure

↓

backup activated successfully

↓

BACKUP_ACTIVE

means recovery is operational.

The primary fault event is resolved after backup operation is confirmed by subsequent telemetry.

So the database doesn't contain a fake:

BACKUP_COOLING_ACTIVE fault

# 23. Stage 13 — Critical failure detection

We then implemented/testing critical thermal failure.

Critical scenario:

chamber_temp >= 12°C

primary = OFF

backup = OFF

Result:

CHAMBER_TEMPERATURE_CRITICAL

severity = CRITICAL

system_state = CRITICAL_FAILURE

Verified example:

Fault ID: 6

Fault:

CHAMBER_TEMPERATURE_CRITICAL

Severity:

CRITICAL

State:

CRITICAL_FAILURE

Resolved:

False

# 24. Critical failure does not issue unsafe cooling command

For a critical chamber condition where both local cooling systems are unavailable, the system doesn't endlessly create:

ACTIVATE_BACKUP

ACTIVATE_BACKUP

ACTIVATE_BACKUP

...

Instead, this condition becomes the trigger for:

Tier-3 rerouting

This separation is important:

Tier 1/2

local recovery

Tier 3

logistics escalation

# 25. Stage 14 — Rerouting database

We added:

rerouting_events

and verified its schema.

Columns:

id

device_id

session_id

triggered_by_fault

source_latitude

source_longitude

destination_id

destination_name

destination_latitude

destination_longitude

distance_km

required_temperature_c

available_capacity_kg

status

data_source

reason

created_at

Defaults include:

status = RECOMMENDED

data_source = SIMULATED

# 26. Rerouting database functions

We added functions equivalent to:

create_rerouting_event(...)

Creates a recommendation.

get_rerouting_event_by_fault(

device_id,

fault_id

)

Checks whether a recommendation already exists for that fault.

get_latest_rerouting_event(

device_id

)

Returns latest recommendation.

get_rerouting_history(

device_id,

limit=100

)

Returns history.

# 27. Stage 15 — Simulated warehouse registry

We created:

backend/services/warehouse_registry.py

Warehouses are explicitly fictional/simulated.

Example:

{

"warehouse_id": "SIM-WH-001",

"name": "Simulated Cold Storage Alpha",

"latitude": 12.9150,

"longitude": 77.6400,

"operational": True,

"min_temperature_c": 2.0,

"max_temperature_c": 8.0,

"available_capacity_kg": 500.0,

"data_source": "SIMULATED"

}

Additional simulated warehouses were created with different:

locations

operational states

temperature ranges

capacities

This lets us test filtering.

# 28. Warehouse compatibility

We implemented:

def is_warehouse_compatible(

warehouse,

required_temperature_c,

required_capacity_kg

):

Logic:

if not warehouse["operational"]:

return False

if warehouse["available_capacity_kg"] < required_capacity_kg:

return False

if not (

warehouse["min_temperature_c"]

<= required_temperature_c

<= warehouse["max_temperature_c"]

):

return False

return True

Current demonstration cargo requirements:

Required storage temperature = 5°C

Required capacity = 100 kg

These are prototype/demo configuration values, not measured cargo requirements.

# 29. Haversine geographic calculation

We implemented a real Haversine distance function:

def haversine_distance_km(

latitude_1,

longitude_1,

latitude_2,

longitude_2

):

Core formula:

earth_radius_km = 6371.0

a = (

sin(delta_lat / 2) ** 2

+

cos(lat1)

* cos(lat2)

* sin(delta_lon / 2) ** 2

)

c = 2 * atan2(

sqrt(a),

sqrt(1 - a)

)

distance = earth_radius_km * c

This gives straight-line great-circle geographic distance.

It is not road-routing distance.

# 30. Destination selection

We implemented:

find_best_destination(...)

Procedure:

Load warehouses

↓

Is operational?

↓

Enough capacity?

↓

Temperature compatible?

↓

Calculate Haversine distance

↓

Sort candidates by distance

↓

Return nearest

Independent test:

python -c "from backend.services.rerouting import find_best_destination; print(find_best_destination(12.9000,77.6500))"

Output:

SIM-WH-001

Simulated Cold Storage Alpha

distance_km = 1.989

So filtering + distance + selection passed independently.

# 31. Stage 16 — Automatic Tier-3 rerouting

We implemented:

evaluate_rerouting(

data,

fault_result

)

It requires:

fault_detected = True

AND

system_state = CRITICAL_FAILURE

AND

fault_id exists

AND

GPS valid

Then:

Check existing rerouting

↓

if existing:

reuse

else:

find destination

↓

store recommendation

# 32. Duplicate rerouting protection

This is a significant part of the implementation.

Without protection:

15 critical packets

↓

15 rerouting rows

would be incorrect.

Instead:

existing = get_rerouting_event_by_fault(

data.device_id,

fault_id

)

if existing is not None:

return existing

So one active fault produces one recommendation.

# 33. Complete critical test

We ran:

python simulator\simulator.py --scenario critical_fault --interval 2

A test session was:

1051e690-610e-48c3-ae54-8e9ef7f36d14

Packets continued up to:

sequence 15

with temperatures around:

13–16°C

and:

Primary = OFF

Backup = OFF

GPS = valid simulated coordinates

Every transmission returned:

API: 200 OK

Stored: True

Transmission: SUCCESS

# 34. Critical database result

We verified:

STATE:

('CCU-001',

'CRITICAL_FAILURE',

'CHAMBER_TEMPERATURE_CRITICAL',

0,

0)

Fault:

(6,

'CHAMBER_TEMPERATURE_CRITICAL',

'CRITICAL',

'CRITICAL_FAILURE',

0)

Rerouting:

(

1,

6,

'SIM-WH-001',

'Simulated Cold Storage Alpha',

1.996,

'RECOMMENDED',

'SIMULATED',

...

)

And most importantly:

TOTAL REROUTES: 1

despite many critical packets.

Therefore:

duplicate rerouting protection = VERIFIED

# 35. Why distance changed from 1.989 to 1.996 km

The independent test used:

12.9000, 77.6500

The actual simulator packet used approximately:

12.89996, 77.650046

Therefore the exact Haversine result changed slightly:

1.989 km → 1.996 km

This is expected because the simulator slightly varies GPS position.

# 36. Stage 17 — Rerouting API

We created:

backend/api/rerouting.py

Latest endpoint:

GET /api/v1/rerouting/{device_id}/latest

History endpoint:

GET /api/v1/rerouting/{device_id}/history

# 37. Latest rerouting API test

Command:

Invoke-RestMethod -Uri "http://127.0.0.1:8000/api/v1/rerouting/CCU-001/latest" | ConvertTo-Json -Depth 10

Verified result:

{

"device_id": "CCU-001",

"rerouting_available": true,

"event": {

"id": 1,

"device_id": "CCU-001",

"triggered_by_fault": 6,

"source_latitude": 12.89996,

"source_longitude": 77.650046,

"destination_id": "SIM-WH-001",

"destination_name": "Simulated Cold Storage Alpha",

"destination_latitude": 12.915,

"destination_longitude": 77.64,

"distance_km": 1.996,

"required_temperature_c": 5.0,

"available_capacity_kg": 500.0,

"status": "RECOMMENDED",

"data_source": "SIMULATED"

}

}

# 38. Rerouting history API test

Command:

Invoke-RestMethod -Uri "http://127.0.0.1:8000/api/v1/rerouting/CCU-001/history" | ConvertTo-Json -Depth 10

Verified:

device_id = CCU-001

count = 1

Therefore:

Rerouting database     PASS

Rerouting engine       PASS

Duplicate prevention   PASS

Latest API             PASS

History API            PASS

# 39. Current telemetry API processing

The current source implements this pipeline:

inserted = save_telemetry(

data=data,

received_at=received_at

)

if inserted:

fault_result = process_fault_lifecycle(

data

)

self_healing_result = evaluate_self_healing(

data=data,

fault_result=fault_result

)

rerouting_result = evaluate_rerouting(

data=data,

fault_result=fault_result

)

The uploaded file confirms the first three stages and the rerouting hook.

The intended final processing chain is therefore:

POST TELEMETRY

↓

Pydantic validation

↓

SQLite storage

↓

Duplicate check

↓

Fault lifecycle

↓

Self-healing

↓

Tier-3 rerouting

↓

JSON response

# 40. Current completed state machine

We can currently demonstrate:

┌──────────┐

│  NORMAL  │

└────┬─────┘

│

abnormal condition

│

▼

┌─────────┐

│ WARNING │

└────┬────┘

│

primary failure

│

▼

┌───────────────┐

│ PRIMARY_FAULT │

└───────┬───────┘

│

self-healing command

│

▼

┌────────────────┐

│ BACKUP_ACTIVE  │

└────────────────┘

If local recovery cannot maintain the chain:

┌──────────────────┐

│ CRITICAL_FAILURE │

└────────┬─────────┘

│

▼

REROUTING

The final specification also defines these core states as the intended self-healing outputs.

# 41. What has been proven so far

At this point we have actual software evidence for:

| Capability | Result |
| --- | --- |
| Python environment | ✅ |
| Project structure | ✅ |
| Digital telemetry simulator | ✅ |
| Multiple fault scenarios | ✅ |
| JSON telemetry | ✅ |
| FastAPI ingestion | ✅ |
| SQLite persistence | ✅ |
| Duplicate telemetry protection | ✅ |
| Rule fault detection | ✅ |
| Fault lifecycle | ✅ |
| Current system state | ✅ |
| Door-fault path | ✅ |
| Primary-current fault | ✅ |
| Self-healing decision | ✅ |
| Control command generation | ✅ |
| Command polling | ✅ |
| Command ACK | ✅ |
| Primary → backup transition | ✅ |
| Backup confirmation | ✅ |
| Critical thermal failure | ✅ |
| Simulated GPS | ✅ |
| Warehouse compatibility | ✅ |
| Capacity filtering | ✅ |
| Temperature filtering | ✅ |
| Haversine distance | ✅ |
| Nearest destination | ✅ |
| Rerouting persistence | ✅ |
| Rerouting duplicate prevention | ✅ |
| Rerouting REST API | ✅ |

# 42. Full demonstrated end-to-end flow

This is the strongest result of the work so far.

### Recovery path

SIMULATED TELEMETRY

↓

FastAPI

↓

SQLite

↓

Fault Detector

↓

PRIMARY_COOLING_CURRENT_FAILURE

↓

PRIMARY_FAULT

↓

Self-Healing Engine

↓

ACTIVATE_BACKUP_COOLING

↓

control_commands

↓

Simulator polls backend

↓

Primary OFF

Backup ON

↓

ACKNOWLEDGED

↓

New telemetry

↓

BACKUP_ACTIVE

### Critical escalation path

SIMULATED TELEMETRY

↓

Chamber temperature > critical threshold

↓

CHAMBER_TEMPERATURE_CRITICAL

↓

CRITICAL_FAILURE

↓

Fault #6

↓

No viable local cooling

↓

Tier-3 Engine

↓

Read simulated GPS

↓

Load simulated warehouses

↓

Filter operational

↓

Filter temperature compatibility

↓

Filter capacity

↓

Haversine calculation

↓

SIM-WH-001 selected

↓

Distance 1.996 km

↓

RECOMMENDED

↓

rerouting_events

↓

REST API

That is a genuine working software-only closed-loop prototype.

# 43. What we have NOT completed

This distinction is essential for the report.

The final project specification requires actual XGBoost and Random Forest inference, predictions/state storage, a dashboard and physical ESP32 functionality.

Currently unfinished:

Professional dashboard

Live dashboard integration

Historical graphs

Map UI

ML dataset preparation

Feature engineering

XGBoost training

Random Forest training

Model evaluation

Ensemble prediction

Prediction database

Predictive-maintenance API

ML → self-healing integration

ESP32-S3 firmware

DS18B20 integration

SHT31 integration

ACS712 integration

Reed switch integration

NEO-6M integration

OLED

LEDs

Buzzer

Peltier physical control

MOSFET physical control

Fan control

Physical interlock verification

Real Wi-Fi telemetry

Real GPS

Real temperature measurements

Real current measurements

Hardware fault injection

Physical backup takeover

Physical cooldown testing

Final validation

The intended ML stage specifically requires real XGBoost and Random Forest implementations, saved trained models, model/version feature schemas and no fabricated prediction probabilities.

# 44. Hardware status

Until physical integration occurs, the following must remain:

PENDING_HARDWARE

We have not yet experimentally measured:

cooldown time

minimum temperature

steady-state temperature

real chamber humidity

real primary current

real backup current

real heatsink temperature

physical failure detection time

physical backup activation time

physical recovery time

physical rerouting activation time

hardware telemetry success rate

The project requirements explicitly say these measurements should not be invented before hardware testing.

# 45. ML status

Currently:

XGBoost                 NOT COMPLETE

Random Forest           NOT COMPLETE

Ensemble                NOT COMPLETE

Predictive API          NOT COMPLETE

Real inference          NOT COMPLETE

Therefore, at this stage, the correct terminology is:

The rule-based self-healing and critical-rerouting software pipeline is operational. Predictive-maintenance ML integration remains under development.

Do not currently say:

“Our AI predicts failures successfully.”

We have not yet generated that evidence.

# 46. Dashboard status

The final dashboard is intended to display chamber temperature, humidity, heatsink temperature, current, door status, primary/backup cooling, GPS, ML probabilities, ensemble risk, system state, live charts, fault timeline, rerouting map and device connectivity.

That remains the next major software block.

# 47. How to run the project currently

### Terminal 1

Open PowerShell:

cd E:\cold_chain_project

.venv\Scripts\Activate.ps1

Start backend:

uvicorn backend.main:app --host 127.0.0.1 --port 8000 --reload

### Terminal 2

cd E:\cold_chain_project

.venv\Scripts\Activate.ps1

Run normal:

python simulator\simulator.py --scenario normal --interval 2

Primary fault:

python simulator\simulator.py --scenario primary_fault --interval 2

Critical:

python simulator\simulator.py --scenario critical_fault --interval 2

Stop:

Ctrl+C

# 48. Database verification

Current system state:

python -c "import sqlite3; c=sqlite3.connect('database/cold_chain.db'); print(c.execute('SELECT * FROM system_state WHERE device_id=?',('CCU-001',)).fetchone()); c.close()"

Fault history:

python -c "import sqlite3; c=sqlite3.connect('database/cold_chain.db'); [print(x) for x in c.execute('SELECT id,fault_type,severity,system_state,resolved FROM fault_events ORDER BY id DESC LIMIT 10').fetchall()]; c.close()"

Rerouting:

python -c "import sqlite3; c=sqlite3.connect('database/cold_chain.db'); [print(x) for x in c.execute('SELECT id,triggered_by_fault,destination_id,destination_name,distance_km,status,data_source FROM rerouting_events ORDER BY id DESC').fetchall()]; c.close()"

# 49. Compile verification procedure

Whenever we edit Python:

python -m py_compile backend\database\db.py

python -m py_compile backend\api\telemetry.py

python -m py_compile backend\api\rerouting.py

python -m py_compile backend\services\fault_detector.py

python -m py_compile backend\services\fault_manager.py

python -m py_compile backend\services\self_healing.py

python -m py_compile backend\services\warehouse_registry.py

python -m py_compile backend\services\rerouting.py

No output means compilation succeeded.

# 50. Current project milestone

I would formally record the current milestone as:

Software Milestone: Rule-Based Closed-Loop Fault Detection, Self-Healing and Tier-3 Emergency Rerouting Completed and Verified in Simulation.

And record the scope as:

Telemetry ingestion                         COMPLETE

Database persistence                        COMPLETE

Rule fault detection                        COMPLETE

Fault lifecycle                             COMPLETE

Self-healing command generation             COMPLETE

Device command simulation                   COMPLETE

Command acknowledgement                     COMPLETE

Backup recovery simulation                  COMPLETE

Critical failure escalation                 COMPLETE

GPS-based simulated rerouting               COMPLETE

Rerouting persistence/API                   COMPLETE

Dashboard                                   PENDING

ML predictive maintenance                   PENDING

ESP32-S3 firmware/physical integration      PENDING

Physical measurements                       PENDING_HARDWARE

Final validation                            PENDING

The overall final project is therefore not complete yet, but the core backend/control foundation is substantial and working.

## Next development sequence

From here I recommend continuing in exactly this order:

Dashboard → ML dataset → feature engineering → XGBoost → Random Forest → model evaluation → ensemble predictor → prediction API/database → integrate ML into self-healing → ESP32-S3 firmware → physical sensor integration → Peltier/MOSFET integration → replace simulator → hardware fault testing → final validation/report.

That will take the current working reactive self-healing prototype and turn it into the intended predictive-maintenance + self-healing + rerouting cold-chain system.